USABILITY FEATURES AND TECH STACK

## IPDS — Reperforation Detection Tool — Usability Feature Document

### CATEGORY A: CAPTURE & SESSION SETUP FEATURES

#### Feature 1: Pre-Flight Capture Checklist

What it does (short): Verifies all required images are present and correctly tagged before the pipeline is allowed to run.

Input:

- Uploaded image set (diffuse/overview, 4 torch-lit photos, backlit/transillumination photo, calibration-target photo, optional ball-bearing photos)

Output:

- Checklist status (Pass/Fail per required item)

- "Process" button enabled/disabled accordingly

How It Works:
On upload, the software checks the tagged image set against the required list defined by the capture protocol (1 diffuse, 4 torch, 1+ backlit, 1 calibration, optional ball-bearing set). Missing or duplicate-tagged items are flagged individually rather than as one generic error.

How It Must Be Implemented:

- Maintain a required-items schema (list of expected tags + min/max counts).

- On each upload event, diff the current session's tagged file set against the schema.

- Disable "Process" until all mandatory items are present; optional items (ball bearing) never block processing.

Requirements:

- Backend: Session Manager, Upload Validation Module

- Frontend: Checklist widget with per-item status icons

#### Feature 2: Structured Session Log Form (with Live Elevation Calculation)

What it does (short): Captures torch geometry and camera settings as structured fields instead of free text, and calculates elevation angle live.

Input:

- Per torch: height (cm), horizontal distance (cm), optional manual angle correction (degrees)

- Camera settings: aperture, shutter speed, ISO, white balance

- Azimuth angle per torch (fixed: 0°/90°/180°/270°)

Output:

- Calculated elevation angle per torch, displayed live

- Structured session log record

How It Works:
As the user types height/distance values for a torch, the form computes elevation = arctan(height ÷ distance) immediately and displays the result, adding any manual correction entered. This mirrors Step 3 (Path A) of the processing pipeline but surfaces it during data entry instead of after the fact.

How It Must Be Implemented:

- Reactive form fields bound to a live calculation function (elevation = atan(height/distance) + correction).

- Store all four torches' final values into the session's log record on save.

- Warn (not block) if calculated elevation falls outside the 15°–20° recommended range.

Requirements:

- Backend: numpy (trigonometric calculation), Session Manager

- Frontend: Form widget with real-time computed field

#### Feature 3: Shadow-Bleed Check Reminder

What it does (short): Reminds the user to perform the manual shadow-bleed check on the physical rig before committing to a torch elevation angle.

Input: None (procedural prompt, not data-driven)

Output: On-screen checklist reminder/confirmation toggle

How It Works:
Before a session is marked "ready to capture," the software displays the shadow-bleed verification step from the hardware protocol (zoom into two adjacent holes at the chosen elevation angle and confirm no shadow overlap) as an explicit checklist item the user must acknowledge.

How It Must Be Implemented:

- Static checklist item tied to session setup stage, gating progression to capture.

- Simple boolean acknowledgment stored in the session log.

Requirements:

- Frontend: Checklist item component

- Backend: none (state only)

#### Feature 4: Brightness-Mismatch Warning

What it does (short): Flags when a torch's brightness-correction factor (Step 4) is unusually far from 1.0, indicating a possible calibration or hardware issue.

Input: Brightness correction factors computed for each of the 4 torch photos (Step 4 output)

Output: Warning indicator per torch, with the factor value shown

How It Works:
After Step 4 computes each torch's correction factor (B_ref / B_i), the value is compared against a tolerance band around 1.0 (e.g., ±0.15). Factors outside this band are visually flagged as a possible hardware issue (dying battery, misplaced torch, ambient light leak).

How It Must Be Implemented:

- Threshold check on the existing Step 4 output — no new computation required.

- Display factor value + warning icon per torch in the session summary.

Requirements:

- Backend: reuse Step 4 (Brightness-Matching) output

- Frontend: Warning badge component

### CATEGORY B: PROCESSING TRANSPARENCY FEATURES

#### Feature 5: Stage-by-Stage Progress Indicator

What it does (short): Shows which processing phase is currently running, labeled by name rather than a generic spinner.

Input: Current pipeline stage (internal state)

Output: Progress bar/stepper labeled by phase (Ingestion → Calibration & Registration → Geometry & Classification → Feature Extraction → Statistical Detection & Output)

How It Works:
The backend pipeline emits a stage-change event each time it transitions between the five phases (and their constituent steps). The frontend subscribes to these events and updates a labeled progress stepper accordingly.

How It Must Be Implemented:

- Pipeline runner emits progress callbacks/events at each of the 23 steps, tagged with phase name.

- Frontend maintains a mapping of step number → phase label, updates UI on each event.

Requirements:

- Backend: Progress event/callback system within the pipeline runner

- Frontend: Stepper/progress bar widget (PySide6 QProgressBar + phase label)

#### Feature 6: Live Thumbnail Preview Per Stage

What it does (short): Displays a thumbnail of each stage's output image as soon as that stage completes, rather than waiting for the full pipeline to finish.

Input: Intermediate image output of each completed stage

Output: Updating thumbnail strip during processing

How It Works:
As each stage (e.g., Step 5 registration, Step 6 boundary detection) writes its output image to disk, the manifest is updated and the frontend polls/subscribes to manifest changes, appending a new thumbnail to the live strip.

How It Must Be Implemented:

- Each pipeline step writes its output image and appends an entry to manifest.json immediately on completion.

- Frontend watches the manifest (file-watch or periodic poll) and refreshes the thumbnail strip incrementally.

Requirements:

- Backend: Manifest writer, file output per stage

- Frontend: Thumbnail strip widget with incremental refresh

#### Feature 7: Per-Hole Progress Counter

What it does (short): Shows numeric progress ("Hole 14/62 processed") during Phase 3, the slowest phase.

Input: Total hole count, current hole index being processed

Output: Live counter text/progress bar

How It Works:
Since Phase 3 (Steps 11–18) runs independently per hole, the pipeline runner tracks total hole count (from Step 8/9 output) and increments a counter as each hole completes its full per-hole feature extraction.

How It Must Be Implemented:

- Pipeline runner maintains holes_processed / total_holes counter, updated after each hole's Step 18 completes.

- Emit this as a progress event to the frontend, same channel as Feature 5.

Requirements:

- Backend: Counter state in pipeline runner

- Frontend: Progress text/bar bound to counter event

### CATEGORY C: VIEW TAB FEATURES

#### Feature 8: Side-by-Side / Overlay Comparison Toggle

What it does (short): Lets the user compare a "before" and "after" image (e.g., raw backlit vs. contour overlay) either side-by-side or as a blended overlay.

Input: Two selected images from the manifest (e.g., raw vs. processed)

Output: Rendered comparison view in chosen mode

How It Works:
The user selects two manifest entries to compare. A toggle switches the display between a side-by-side split view and an alpha-blended overlay view of the same two images, aligned pixel-for-pixel (images are already registered per Step 5).

How It Must Be Implemented:

- Since all session images are already co-registered (Step 5), no re-alignment is needed at display time.

- Side-by-side: render both images in adjacent panes.

- Overlay: render one image with adjustable alpha blend over the other.

Requirements:

- Frontend: QGraphicsView/QGraphicsScene with dual-image compositing, alpha slider control

#### Feature 9: Synchronized Zoom/Pan

What it does (short): Keeps zoom and pan position identical across two compared images.

Input: Zoom/pan interaction on one image pane

Output: Mirrored zoom/pan applied to the paired image pane

How It Works:
Mouse-wheel zoom and drag-pan events on one QGraphicsView are captured and the same transform (scale + translation) is applied to the second view's transform matrix, since both images share the same coordinate frame post-registration.

How It Must Be Implemented:

- Bind both QGraphicsView widgets' transformation matrices together via a shared signal/slot connection.

- On any transform change in one view, propagate identical transform to the other.

Requirements:

- Frontend: PySide6 QGraphicsView signal/slot linkage

#### Feature 10: Filter/Group Controls (View Tab)

What it does (short): Lets the user filter or group the image gallery by pipeline stage, hole ID, edge, or flagged status.

Input: Filter selection (stage / hole / edge / flagged-only)

Output: Filtered gallery view

How It Works:
The manifest already tags every image with stage, hole ID (where applicable), edge, and outlier flag (once statistics complete). The gallery view applies a filter predicate over manifest entries and re-renders only matching thumbnails.

How It Must Be Implemented:

- Manifest schema includes stage, hole_id, edge, flagged fields per entry.

- Frontend filter widget applies a predicate function over the in-memory manifest list.

Requirements:

- Backend: Manifest tagging fields (already required by Feature 6)

- Frontend: Filter/dropdown control bound to gallery re-render

#### Feature 11: Hover Tooltips With Descriptor Values

What it does (short): Shows the actual computed numbers (shape/roughness/shadow values) when hovering over a hole thumbnail.

Input: Hole's composite descriptor (Step 18 output)

Output: Tooltip text listing key descriptor values

How It Works:
On mouse-hover over a hole thumbnail, the frontend looks up that hole's descriptor record and renders a formatted tooltip with the relevant numeric fields, avoiding reliance on color-coding alone.

How It Must Be Implemented:

- Attach hole descriptor data to each thumbnail widget as metadata.

- Standard tooltip event handler formats and displays the values on hover.

Requirements:

- Frontend: Tooltip component bound to thumbnail widgets

### CATEGORY D: MATH EXPLANATION & TRACEABILITY FEATURES

#### Feature 12: Per-Hole "Why Flagged" Breakdown Panel

What it does (short): For any selected hole, shows its comparison group, per-descriptor values vs. group statistics, and which descriptor(s) drove the outlier flag.

Input: Selected hole ID

Output: Table of {descriptor name, hole value, group median, group MAD, robust z-score}; highlighted driver descriptor(s)

How It Works:
Uses the existing Step 19 (reference-group construction) and Step 20 (robust z-score) outputs. Rather than collapsing them into one combined score, the panel lists every individual descriptor's z-score, and visually highlights whichever descriptor(s) exceed the outlier threshold, so the user can see which signal (shape, roughness, or shadow) is responsible.

How It Must Be Implemented:

- Pull per-descriptor median/MAD/z-score values already computed in Step 20 (do not recompute).

- Sort/highlight rows by z-score magnitude.

- Display comparison-group description text (edge-based vs. pooled fallback, with low-confidence flag if applicable, per Step 19).

Requirements:

- Backend: expose Step 20's per-descriptor intermediate values (not just the combined score) in the output structure

- Frontend: Table widget with conditional row highlighting

#### Feature 13: Pairwise Hole Comparison View

What it does (short): Compares a flagged hole directly against a specific reference hole (auto-selected as the group's median-closest hole, with manual override).

Input: Selected hole ID; optional manually-selected comparison hole ID

Output: Side-by-side descriptor value table + paired image crops/overlays

How It Works:
By default, the system identifies the comparison group's most "typical" hole (smallest combined deviation from the group median) and displays it alongside the selected hole — matching crops from the View tab, plus a table of each descriptor's raw values and the delta between the two holes. A manual dropdown allows overriding the auto-selected reference hole with any other hole in the session.

How It Must Be Implemented:

- Auto-selection: compute each comparison-group hole's combined deviation score against the group median (reuses Step 20 logic on the group holes themselves); pick the minimum.

- Manual override: dropdown populated from all holes in the current session.

- Render both holes' descriptor tables and crops side by side with delta column.

Requirements:

- Backend: extend Step 20 group logic to also rank in-group holes by centrality

- Frontend: Dual-pane comparison layout + dropdown selector

#### Feature 14: Shadow-Direction Vector Diagram

What it does (short): Visualizes each hole's shadow direction and strength as an arrow on a shared compass plot, so directional inconsistency is visible at a glance.

Input: Shadow strength + direction (Step 17 output) for a set of holes (typically one edge/group)

Output: Compass-style vector plot, one arrow per hole

How It Works:
Each hole's Step 17 output (shadow strength = arrow length, shadow angle = arrow direction) is plotted as a vector from a common origin on a polar/compass diagram. Holes whose arrows point noticeably differently from the group's dominant direction are visually distinguishable without reading any numbers.

How It Must Be Implemented:

- Use existing Step 17 outputs directly (no recomputation).

- Render via Matplotlib polar plot embedded in the Qt view (FigureCanvasQTAgg).

- Optionally color-code arrows by outlier flag status.

Requirements:

- Backend: Step 17 outputs (strength, angle) per hole

- Frontend: Matplotlib polar plot embedded via FigureCanvasQTAgg

#### Feature 15: "Show the Math" Expandable Explanation Blocks

What it does (short): Collapsible per-descriptor sections that render the actual formula populated with that hole's real computed numbers, in plain language.

Input: Selected hole's descriptor values and its comparison group's statistics

Output: Expandable text block per descriptor (shape, roughness, shadow) with formula + real values + a plain-language interpretation sentence

How It Works:
For each descriptor category, a template string is populated with that specific hole's actual numbers (e.g., "GLCM contrast: 18.4 vs. group median 6.1, MAD 1.8 → robust z = 5.2"), followed by a one-line plain-language interpretation. This is generated live from already-computed Step 13/15/17/20 values — not a static explanation.

How It Must Be Implemented:

- Define one text template per descriptor type with placeholders for hole value / group median / group MAD / z-score.

- Populate templates at render time from the hole's existing computed data.

- Nest inside a collapsible/expandable Qt widget (QToolBox or collapsible QFrame).

Requirements:

- Backend: none beyond existing descriptor outputs

- Frontend: Collapsible panel widget, template-based text rendering

#### Feature 16: Edge-Level Consistency Summary

What it does (short): One combined view per edge showing all holes' shadow-direction arrows and roughness scores together, surfacing clustering patterns visually.

Input: All holes' shadow and roughness descriptors for one edge

Output: Combined compass plot (direction) + strip/scatter plot (roughness) for that edge

How It Works:
Aggregates Feature 14's vector diagram with a simple 1D strip plot of roughness scores across all holes on one edge, positioned in hole-order. This lets the pattern that Step 22 detects computationally (a run of flagged holes) be seen visually as a cluster, rather than only reported as a text region.

How It Must Be Implemented:

- Reuse Step 17 (shadow) and Step 15 (roughness) outputs, grouped by edge (Step 8 tag).

- Render two synchronized subplots: polar vector plot + linear strip plot, ordered by position-along-edge.

Requirements:

- Backend: existing per-hole outputs grouped by edge

- Frontend: Matplotlib dual-subplot figure embedded via FigureCanvasQTAgg

#### Feature 17: Validated-Source Info Links

What it does (short): Small info icons next to each descriptor's explanation linking to the specific prior-art/literature source backing that descriptor.

Input: Descriptor type being displayed

Output: Info icon → popover/tooltip with citation text and source reference

How It Works:
Each descriptor type (shape/circularity, roughness, directional shadow, leave-one-out statistics) has a fixed citation record (drawn from the project's compiled literature review) attached in a lookup table. The info icon displays this record on click/hover.

How It Must Be Implemented:

- Static lookup table: descriptor_type → {citation_text, source_reference}.

- Info icon triggers a popover rendering the matched entry.

Requirements:

- Backend: static citation reference table (from project literature review)

- Frontend: Info icon + popover component

### CATEGORY E: RESULTS & REPORTING FEATURES

#### Feature 18: Confidence-Framed Results Display

What it does (short): Presents results as a suspicion score with supporting evidence rather than a binary "fake/real" verdict.

Input: Final per-hole scores, outlier flags, candidate region(s) (Step 21–22 output)

Output: Results screen framed as "candidate flagged for review" with confidence/severity level, never a hard verdict

How It Works:
The results screen deliberately avoids binary "FAKE"/"GENUINE" language. Instead, it presents a suspicion level (e.g., Low/Medium/High, derived from the deviation score's distance past threshold) alongside the specific evidence (which holes, which descriptors, which region) supporting that level.

How It Must Be Implemented:

- Map combined deviation score ranges to descriptive suspicion bands (not pass/fail).

- Always display the underlying evidence table alongside the summary label — never show the label in isolation.

Requirements:

- Backend: Step 21/22 outputs

- Frontend: Results summary component with banded severity display

#### Feature 19: "Why This Region Was Flagged" Summary

What it does (short): Per candidate region (Step 22), shows which descriptor(s) drove the flags across that whole cluster of holes.

Input: Candidate region's hole list and their per-descriptor z-scores

Output: Region-level summary panel

How It Works:
Aggregates Feature 12's per-hole breakdown across all holes in a flagged region, identifying which descriptor(s) are consistently elevated across the cluster — distinguishing, e.g., a region flagged primarily on shadow-direction inconsistency from one flagged on roughness.

How It Must Be Implemented:

- For each region (Step 22 output), average/aggregate the per-descriptor z-scores across its member holes.

- Display the dominant descriptor(s) prominently.

Requirements:

- Backend: Step 22 region data + Step 20 per-descriptor scores

- Frontend: Region summary panel

#### Feature 20: Exportable PDF Report

What it does (short): Generates a single PDF bundling the annotated overview image and the full structured results table.

Input: Final annotated image (Step 23), structured hole data table (Step 23)

Output: PDF file

How It Works:
Combines the annotated overview image, a legend, the full per-hole results table, and summary/region findings into one formatted document suitable for attaching to a case file or collection record.

How It Must Be Implemented:

- Use a PDF generation library to lay out: title/session metadata → annotated image → results table → region summary.

- Pull data directly from existing Step 23 outputs; no new computation.

Requirements:

- Backend: PDF generation library (e.g., ReportLab or WeasyPrint)

#### Feature 21: Low-Confidence Tagging Surfaced in UI

What it does (short): Visually distinguishes holes/results that fell back to pooled (whole-specimen) comparison due to low per-edge hole count.

Input: low_confidence flag per hole (Step 19 fallback output)

Output: Distinct visual marker (icon/color) wherever that hole appears in any view

How It Works:
Wherever a hole is displayed (gallery, breakdown panel, results table, region summary), if its low_confidence flag is set, a consistent visual marker is shown so the reduced reliability of that result is never hidden behind an ordinary-looking flag.

How It Must Be Implemented:

- Single shared flag-rendering component/style used across every view that displays hole-level results.

- Enforce this as a global rendering rule rather than a per-view special case.

Requirements:

- Backend: low_confidence field from Step 19

- Frontend: Shared flag/badge component

### CATEGORY F: SESSION CONTINUITY FEATURES

#### Feature 22: Crash-Persistent Session (Resume on Launch)

What it does (short): Detects an incomplete/crashed session on application startup and offers to resume it.

Input: manifest.json found on disk at startup

Output: Resume prompt, or fresh session if none found

How It Works:
On launch, the application checks the session directory for an existing manifest.json. If found and marked incomplete, the user is prompted to resume (reloading all recorded state and completed-stage images) or discard and start fresh.

How It Must Be Implemented:

- manifest.json is the single source of truth for session state, written incrementally at every stage (per Feature 6).

- On startup, check for its existence and status field; if incomplete, show resume dialog; reload state from the manifest if resumed.

Requirements:

- Backend: Manifest-based session state (already required by other features)

- Frontend: Resume/discard dialog on startup

#### Feature 23: Session Reset With Confirmation

What it does (short): Wipes the current session's images and state from disk, but only after explicit user confirmation.

Input: User-initiated "Reset Session" action

Output: Empty session state; disk files removed

How It Works:
Clicking "Reset" triggers a confirmation dialog warning that this action is irreversible. Only on explicit confirmation does the software delete the session's disk folder and clear in-memory/manifest state.

How It Must Be Implemented:

- Confirmation dialog (QMessageBox) before executing deletion.

- On confirm: shutil.rmtree() the session directory, reset in-memory session object.

Requirements:

- Backend: Session Manager (delete operation)

- Frontend: Confirmation dialog

#### Feature 24: Reprocess-From-Stage

What it does (short): Allows re-running the pipeline from a specific stage onward, without re-uploading or re-running from scratch.

Input: Selected stage to resume from; corrected input for that stage if applicable (e.g., recalibrated values)

Output: Updated pipeline outputs from that stage forward

How It Works:
Since each stage's inputs/outputs are already tracked in the manifest, the user can select an earlier stage (e.g., "redo from Calibration"), the software discards manifest entries from that stage onward, and reruns only those steps using the still-valid outputs of earlier stages as input.

How It Must Be Implemented:

- Pipeline runner must support starting execution at an arbitrary step index, given valid prior-stage outputs already on disk.

- On reprocess request, invalidate/remove manifest entries at and after the selected stage, then re-invoke the runner from there.

Requirements:

- Backend: Pipeline runner with resumable/stage-addressable execution

- Frontend: Stage-selection control for reprocessing

### CATEGORY G: VALIDATION & PATENT-SUPPORT FEATURES

#### Feature 25: Parameter/Threshold Settings Panel

What it does (short): Displays all thresholds and constants used in the current run (outlier cutoff, MAD scale factor, wave-fit order, etc.), visibly rather than hidden.

Input: None (reads current configuration)

Output: Read-only settings summary panel

How It Works:
All tunable constants used across the pipeline (Step 20's 0.6745 MAD scale constant, Step 21's cutoff value, Step 13's Fourier order, Step 19's minimum-holes-per-edge fallback threshold) are stored in one configuration object and displayed together, so results are reproducible and defensible.

How It Must Be Implemented:

- Centralize all pipeline constants into a single config object/file rather than scattering literals through code.

- Render this config as a read-only panel accessible from the results view.

Requirements:

- Backend: Central pipeline configuration module

- Frontend: Settings display panel

#### Feature 26: Validation Mode (Labeled Specimen Benchmarking)

What it does (short): Runs the full pipeline against known-genuine and known-altered validation specimens and reports detection accuracy and repeatability.

Input: Labeled specimen set (genuine reference; hollow-punch, punch-plier, prong-punch, pen/screwdriver negative controls)

Output: Per-specimen detection accuracy report; simple ROC-style summary across threshold values; inter-session repeatability report (same specimen run twice)

How It Works:
A dedicated mode runs the standard pipeline against each labeled specimen in the validation set, compares the software's flagged holes against the known ground-truth alteration location, and aggregates true/false positive rates. For repeatability, the same specimen is processed in two independently-captured sessions and the resulting scores/flags are compared for drift.

How It Must Be Implemented:

- Accept a "ground truth" label per validation specimen (which holes/edges are known-altered).

- Run pipeline normally; compare output flags against ground truth to compute accuracy metrics.

- For repeatability: run pipeline on two sessions of the same specimen; compute score deltas per matched hole.

- Sweep Step 21's cutoff across a range of values to produce a simple ROC-style table/plot.

Requirements:

- Backend: Validation Engine (ground-truth comparison logic), reuses full pipeline

- Frontend: Validation Mode screen with accuracy/repeatability report display

#### Feature 27: Full Session Export/Archive

What it does (short): Bundles all raw images, intermediate outputs, manifest, descriptor tables, and session log into one archive for reproducibility record-keeping.

Input: Completed session (manifest + all associated files)

Output: Single timestamped archive folder/zip file

How It Works:
On request, the software copies (or zips) the entire session directory — raw uploads, every intermediate stage image, the manifest, the final results table, and the session log — into one self-contained, timestamped package.

How It Must Be Implemented:

- Simple directory copy/zip operation over the existing session folder (already fully populated via the manifest-driven design).

- Name the archive with specimen ID + timestamp for traceability.

Requirements:

- Backend: Archive/export utility (shutil.make_archive or equivalent)

#### Feature 28: Method/Version Stamping

What it does (short): Records pipeline version, threshold constants, and library versions on every exported output for traceability.

Input: None (reads build/environment metadata)

Output: Version/metadata block embedded in every export (PDF report, archive, manifest)

How It Works:
Every export operation (Feature 20, Feature 27) automatically includes a metadata block recording: pipeline code version/commit hash, the exact constants from Feature 25's config at time of run, and versions of key libraries (OpenCV, scikit-image, numpy, scipy).

How It Must Be Implemented:

- Capture version info at pipeline start (library __version__ attributes, application version constant, optionally git commit hash).

- Attach this metadata block to the manifest and propagate it into every export format.

Requirements:

- Backend: Version-capture utility, embedded into Manifest writer and Export modules

#### Feature 29: Input-Quality Gating

What it does (short): Checks basic capture quality (backlit image contrast, torch exposure consistency, calibration target detection) before allowing processing to proceed.

Input: Uploaded raw images (pre-processing)

Output: Quality check pass/fail per criterion, with warnings if marginal

How It Works:
Before the pipeline runs, lightweight checks assess whether the backlit image shows the expected bimodal (bright hole / dark paper) brightness distribution, whether the four torch images have comparable overall exposure, and whether the calibration target is detectable at all. Failures block processing; marginal results produce a warning the user must acknowledge.

How It Must Be Implemented:

- Backlit check: compute image histogram, verify clear bimodal separation (e.g., via Otsu threshold quality/variance check).

- Torch exposure check: compare mean brightness across the 4 torch images within a tolerance band.

- Calibration check: attempt Step 2's pattern detection early; fail gate if not found.

Requirements:

- Backend: Lightweight pre-processing quality-check module (reuses parts of Steps 2, 4, 7 logic ahead of full pipeline execution)

- Frontend: Quality gate results panel with pass/fail/warning indicators

## IPDS — Confirmed Technology Stack

| Category | Technology / Package | Purpose in Project |
| --- | --- | --- |
| Application Type | Standalone desktop application (no client-server/network layer) | Runs entirely on one local machine; no browser, no server process exposed to network |
| Core Language | Python | Single language across UI and processing pipeline |
| Frontend/UI Framework | PySide6 (Qt for Python) | Main application window, tabs (Upload/Processing/View/Results/Validation), forms, dialogs |
| UI Theming | Qt Style Sheets (QSS); optionally qt-material or PyQtDarkTheme | Custom modern look (dark theme, accent colors, card-style panels) |
| Image Display/Interaction | QGraphicsView / QGraphicsScene (part of PySide6) | Pan/zoom, overlay compositing, synchronized zoom/pan, side-by-side comparison |
| Plotting/Visualization | Matplotlib, embedded via FigureCanvasQTAgg | Anomaly heatmaps, shadow-direction vector/compass plots, roughness strip plots, edge-consistency summaries |
| RAW Image Decoding | rawpy (LibRaw-based) | Decoding camera RAW files into linear numeric arrays (Step 1) |
| Core Image Processing | OpenCV (cv2) | Lens calibration/undistortion, contour detection, connected-component blob detection, image registration (ECC/ORB+homography), moments-based centroid calculation |
| Subpixel Contour Extraction | scikit-image (skimage.measure.find_contours) | Subpixel hole boundary extraction (Step 11) |
| Texture Analysis | scikit-image (skimage.filters.sobel, skimage.feature.graycomatrix/graycoprops) | Roughness descriptor computation (Step 15) — gradient and GLCM texture statistics |
| Numerical Computing | NumPy | Array operations, brightness calculations, angular binning, vector/circular averaging |
| Signal Processing / Curve Fitting | SciPy (scipy.optimize, scipy.stats, scipy.ndimage) | Fourier/wave fitting (shape descriptor), circle fitting for teeth, least-squares directional profile fitting (Step 17), median/MAD statistics, spatial clustering (run detection) |
| Robust Statistics | scikit-learn (MinCovDet, conceptually RobustScaler) | Robust covariance estimation for Mahalanobis-distance-based combined deviation scoring (Step 20) |
| Geometry Utilities | Shapely (or plain OpenCV geometry as alternative) | Point-in-shape / distance-to-boundary tests for edge/tooth classification (Step 8) |
| Session State / Data Persistence | Local filesystem + manifest.json (no database) | Session state, per-stage image paths, descriptor data, crash-persistence, resumability |
| File I/O for Logs/Tables | Python built-in json/csv, optionally pandas | Session log parsing, structured results table generation |
| PDF Report Generation | ReportLab or WeasyPrint | Exportable PDF report bundling annotated image + results table |
| Session Archiving/Export | Python built-in shutil (shutil.make_archive) | Full session export/archive feature |
| Version/Metadata Capture | Library __version__ attributes; optionally gitpython or subprocess for commit hash | Method/version stamping on exports |
| Packaging (Prototype → .exe, future) | PyInstaller | Bundling the PySide6 app into a standalone executable |
| Testing (recommended, not yet built) | pytest | Unit/integration testing of pipeline stages |